A:              /3 B :             /6  C:          /6 Tot:        /15

Laboratory 3
Negative Feedback in Opamp applications

Introduction

We will see today how to employ negative feedback to control the gain of an opamp working in amplification mode. In the Lab 2 exercise of building a Schmitt trigger, the opamp was working in switching mode: Vo switches alternately between positive and negative saturation. Now we would like to use it as a linear amplifying device with output vo= G∙vin where G is a value set by the designer. 
Note that G is different from the intrinsic open-loop gain A of the amplifier. A~106 is a characteristic of the device that you cannot change. We will build circuit connections around the opamp to make it a linear amplifier with gain G.

Part A: Simple Negative Feedback

Consider the circuit shown in Fig 1. It implements a simple negative feedback loop. Feedback is negative when some fraction of the output is subtracted from the input.

vo

vin

Fig 1: Basic negative feedback

/2 mks

/1 mk

Using Kirchoff’s laws, obtain the relation between vo and vin  and hence the gain G of this circuit.

Warmup: Setup the circuit of Fig 1 your breadboard to make sure that your basic circuit is working. Use vin as a sine wave of frequency 1 kHz and suitable amplitude. Note carefully where you connect +Vcc , -Vcc ,0V  and vin ,vo connections. Earth ground is typically not used in opamp circuits.

- Demonstrate vout and vin on the DSO as two time traces.

- Measure the phase difference (if any) between vo & vin. Figure out a simple method of measuring the phase difference since you will be making many phase measurements in the next part of the lab.

Let’s call the phase difference Δφ. Measured Δφ =                         (in degrees)

Part B: Negative Feedback with gain G

We would like to modify the circuit of Fig 1 to have a gain G=10. Draw here the circuit diagram of a negative feedback circuit you would design to set G=10.

Hint:As discussed in lecture, this is a linear circuit and requires adding just two passive components to the circuit of Fig 1
Q1: What is the input impedance of your G=10 circuit? (calculated)
       You can try measuring the DC value of the input impedance (i.e. resistance) using the DMM

/0.5 mk

/0.5 mk

/0.5 mk

/0.5 mk

Q2: As in Part A, setup your circuit on the breadboard. Use vin as a sine wave of frequency 1kHz.

A: Demonstrate the time traces of vin and vout .Note that G=10 and the maximum allowed vo=±Vmax places restrictions on the amplitude of vin

B: Running the circuit with Vcc=±12V and G=10, what is the maximum vin the circuit can theoretically linearly amplify without hitting the saturation limits vo=±Vmax?
vin|max = _________ 
Set your vin at 0.1 × vin|max

C: Measure the phase difference Δφ=_________ degrees for vin amplitude at 0.1×vin|max

Q3: Measure how the gain and phase difference change as you increase the frequency of vin You may mark your observations on the following plot. Note that the x axis on both plots is logarithmic. The values for y axis on the gain plot are already marked. Label the y axis values on the phase difference Δφ plot as per your measurements.

A):  Measure gain and Δφ at frequencies 100 Hz, 1kHz, 10 kHz, 100 kHz and 1 MHz

102

103

104

105

106

frequency (Hz)→

(Hz)→

Phase difference Δφ (degrees) →

0.01

0.1

1

10

102

103

104

105

106

frequency (Hz)→

Gain G →

. Keep vin amplitude set at one tenth of vin|max

/2 mks

/2 mks

B) For one low frequency vin where G=10 (eg. 1 kHz) increase vin  to vin|max . 
 You would expect to see linear amplification with G=10 since vo would be within the limits of ±Vmax Draw a sketch of your observations here indicating the amplitudes and signal shapes.

Provide an explanation of your observation here:________________________________________

________________________________________________________________________________

Part C: Load Test to be performed at reduced vin = 0.1× vin|max

As discussed in the lecture, we have built a Voltage Controlled Voltage Source (VCVS). An ideal voltage source provides voltage to the load irrespective of the value of load resistance. We will test the limits of ‘ideal’ behavior of our negative feedback circuit.

So far you have been using the DSO probe directly at the output of the LM741. The DSO probe has very large impedance .

Connect the following values of RL between vo and 0V. Make a sketch of vo(t) for each, noting the amplitudes:

- RL = 1000 Ω

- RL = 100 Ω

RL = 50 Ω

/3.5 mks

/2.5 mks

- RL = 25 Ω (eg. 50 Ω || 50 Ω)

Look up the LM741 datasheet on your desktop PC – it specifies the short circuit output current for the opamp. This is the maximum current the opamp can supply when RL = 0 Ω 
With this information explain your observations 1,2,3,4 above. 
vo = G × vin and our vin is set at  less than vin|max  so in principle we expect vo to swing linearly in the full range up to ±Vmax. If this is not the case, provide a quantitative explanation for any deviation observed from the expectation.