A:           /5B:              /4C:         /6 Tot:        /15

Laboratory 8

Relaxation Oscillators

This lab consists of three modules. We will see how parasitic effects can occasionally be used to a designer’s advantage

Part A:  Capacitor connected at Input

Fig 1: Cs at source

Cs

Rf

Consider the circuit of Fig 1. Notice the difference from standard inverting feedback amplifier: the

/1

/2

/2

source resistor has been replaced by a capacitor Cs.

Q1:Calculate  the time-domain response of Vout to Vin as in Part A. The time-domain response requires you to apply the golden rules of opamp design and the current-voltage relationship for a capacitor I = C dV/dt followed by some algebra. 
Current at (-) input : Vo/R + C dVin/dt = 0   so Vo = - RCdVin/dt   → Differentiator (with – sign)

Q2: Choose suitable values of the passive components Rs ,Cf such that the time constant of your design is 1 ms.

Q3: Build the circuit of Fig 2 and drive it with a square Vin of amplitude 2V peak-to-peak. 
Note that Cs is in the signal path, so the type of capacitor used is important. Choose wisely!
Draw a diagram of your observations here and verify that your calculations of Q1 above are correct.

Square differentiate → flat 0 except +/- delta functions at edges
Delta function → opamp output saturates and rings after it comes out of saturation.
Can fix this by adding Rf/2 in series with Cs to set a very low gain.

Part B:  Comparator
Recall the simple comparator (with hysteresis) circuit using positive feedback we studied earlier in Lab 2. The function of a comparator is to feedback a fraction of Vout to the (+) input so that when Vin exceeds a threshold set by the feedback fraction Rf+/R+, the output switches to -Vsat and when Vin falls below threshold, output switches to +Vsat

Use your memory/logic and draw the diagram of a simple comparator circuit here:

Fig 2: Simple comparator with positive feedback. Set the reference voltage for comparison to Vsat *(R+/ Rf++ R+)

Rf+

R+

-
+

+

Q1: For Vin= 0, predict the values of Vout in your circuit:

          a) What is Vout at t=0 immediately when the circuit is turned on? 
               0V

/0.5

b)  For an ideal power supply that turns on +/-Vcc as a step function,  what is Vout in the 
                very short time ~ microseconds after the circuit is turned on?

/1

/0.5

/0.5

Rises to + or - Vsat limited by the slew rate ~ 0.1/μs : so it will take ~ 10 μs to reach Vsat

c) What is Vout in steady state after the circuit has been on for a few seconds: 
               +/- Vsat

/0.5

d) What determines the steady state value of Vout in question c) ?

Sign of input offset voltage Vos
        

Q2: Connect up the circuit you have designed in Fig 2 and demonstrate the validity of your predictions
Use this space to record your DSO observations of Vout in detail. 
Note that Vin = 0 for a), b), c)
Your observations for (b) may differ from prediction because the power supply is not ideal. Observe the voltage +/-Vcc supplied by the power supply to the circuit at t=0 on the DSO to validate your prediction.

/2

Part C:  Relaxation oscillator

Use your observations and conclusions from Parts A and B to make a circuit with one opamp that demonstrates the following behavior:

1.  It’s Vout oscillates between two fixed values (for example +Vsat and -Vsat )
2.  It has no external Vin applied 
3. The oscillation time period is 1 ms

Hint: You will need to combine the concepts of Parts A and B

Draw your circuit design here. Calculate and mark the value of the components required.

Comparator with differentiator in negative feedback loop
Relies on delta function created by  0 to +Vsat switching to force output to switch to –Vsat and vice-versa

/3.5

/2.5

This type of oscillator is called a ‘Relaxation Oscillator’. Build your circuit and demonstrate its operation